SECTION 101200 - DISPLAY CASES

TIPS:

To toggle non-printing Editor's Notes that provide guidance for editing, click on "File", "Options", "Display", then select "Hidden Text".

Revise this Section by deleting and inserting text to meet Project-specific requirements.  Where options are indicated by use of brackets, i.e. [ ], delete options not applicable to this project.

- GENERAL

      - RELATED DOCUMENTS

Retain or delete this article in all Sections of Project Manual.

        - Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

      - SUMMARY

        - Section Includes:

          - Recessed Display Case with Sliding Glass Doors.

        - Related Requirements:

Retain subparagraphs below to cross-reference requirements Contractor might expect to find in this Section but are specified in other Sections.

          - Section 097723 "Fabric-Wrapped Panels" for tackable, fabric-covered wall panels.

          - Section 101100 "Visual Display Units" for tackboards.

          - Section 101300 "Directories" for boards with changeable messages or changeable letters and for bulletin boards within built-in directories.

      - DEFINITIONS

Retain terms that remain after this Section has been edited for a project.

        - Display Case: Glazed cabinet with tackboard panel back surface and adjustable shelves.

        - Tackboard Panel: A material for holding push-pins or tacks typically consisting of a facing; such as fabric, vinyl, or cork; adhered to a substrate; such as fiberboard, hardboard, particleboard.

If needed, insert list of conference participants not mentioned in Section 013100 "Project Management and Coordination."

      - ACTION SUBMITTALS

        - Product Data: For each type of product.

          - Include construction details, material descriptions, dimensions of individual components and profiles, and finishes for display cases. Include furnished specialties and accessories.

          - Include electrical characteristics for illuminated display cases.

        - Shop Drawings: For bulletin boards.

          - Include plans, elevations, sections, and attachment details.

          - Show location of seams and joints in tackboard panels.

          - Include sections of typical trim members.

          - Include diagrams for wiring of illuminated display cases.

        - Samples: For each exposed product and for each color and texture specified; not less than 8-1/2 by 11 inches (215 by 280 mm) for tackboard panels and 6 inches (150 mm) long for trim with factory finish.

        - Samples for Initial Selection: For each type of exposed finish.

          - Include Samples of tackboard panels and factory-finished trim involving color finish selection.

        - Samples for Verification: For each type of exposed finish for the following.

          - Tackboard Panel: Not less than 8-1/2 by 11 inches (215 by 280 mm), with facing and substrate indicated for final Work. Include one panel for each type, color, and texture required.

          - Trim: 6-inch- (150-mm-) long sections of each trim profile including corner section.

      - CLOSEOUT SUBMITTALS

        - Maintenance Data: For bulletin boards to include in maintenance manuals.

      - FIELD CONDITIONS

        - Environmental Limitations: Do not deliver or install bulletin boards for indoor installations until spaces are enclosed and weathertight, wet-work in spaces is complete and dry, work above ceilings is complete, and temporary HVAC system is operating and maintaining ambient temperature and humidity conditions at occupancy levels during the remainder of the construction period.

- PRODUCTS

      - MANUFACTURERS

        - Source Limitations: Obtain bulletin boards from single source from single manufacturer.

      - PERFORMANCE REQUIREMENTS

Retain "Surface-Burning Characteristics" Paragraph below if tackboard panels or facings are flammable and testing is required by authorities having jurisdiction.

        - Surface-Burning Characteristics: Comply with ASTM E84; testing by a qualified testing agency. Identify products with appropriate markings of applicable testing agency.

          - Flame-Spread Index: 25 or less.

          - Smoke-Developed Index: 50 or less.

- Delete the following if lighted cases are not required

        - Electrical Components: Listed and labeled as defined in NFPA 70, by a qualified testing agency, and marked for intended location and application

      - DISPLAY CASE

Copy this article and re-edit for each product.

Insert drawing designation. Use these designations on Drawings to identify each product.

        - Display Cases <Insert drawing designation>.

          - Basis of Design: Subject to compliance with the requirements, provide Platinum Visual Solutions "Recessed Display Case (HDS)" framed display case with sliding glass doors.

          - Prior to receipt of bids, additional Manufacturer’s whose products equal or exceed the specified products in every regard to the specified products may request to be included as a specified manufacture by following procedures outlined in Division 01 Section "Substitution Procedures".

        - General: Factory-fabricated unit consisting of manufacturer's standard wall-mounted cabinet with tackboard panel on back inside surface and operable glazed doors at front.

          - Frame and Cabinet Profile: Square frame section with square cabinet corners.

          - Mounting: Surface mounted.

          - Size: As indicated on Drawings.

        - Recessed Display Case: Factory-fabricated display case; with finished interior, operable glazed doors at front, and trim on face to cover edge of recessed opening.

          - Display Case Cabinet: Hardwood veneer plywood.

Select "Veneer Species" or "HPL plastic laminate" Subparagraph below for wood cabinet box.

            - Veneer Species: [Birch veneer with stained finish] [HPL plastic laminate].

          - Face Frame: Aluminum.

        - Aluminum Frames and Trim: Fabricated from not less than 0.062-inch- (1.57-mm-) thick, extruded Type 6063 aluminum; Type HS600 standard frame with 1-3/4 inch face.

Select "Aluminum Finish" from options below for finish of aluminum components.

          - Aluminum Finish: [Clear anodic] [Color anodic] [Manufacturer's standard baked enamel or powder coat].

            - Color: As selected by Architect from manufacturer's full range.

        - Sliding Doors: Tempered glass; set in frame matching cabinet material and finish. Equip each door with two keys.

          - Glass Thickness: Not less than 6 mm thick.

          - Housing AT220: One piece aluminum 2" x 2" angle perimeter housing.

          - Door Frame CH200: Channel frame with plastic seal at glass. Channel to have integrated wheels.

          - Top Door Track TT760: One piece aluminum channel with plastic guides at top of glass.

          - Bottom Door Track BT660: One piece aluminum channel with roller guide.

          - Tumbler lock; Sliding ratchet lock; keyed alike as applicable.

        - Shelves: 6-mm-thick tempered glass; supported on adjustable shelf standards and supports.

          - Shelf Depth: [6 inches (150 mm)] [8 inches (200 mm)] [10 inches (250 mm)] [12 inches (300 mm)].

          - Number of Shelves: As indicated on Drawings.

First option in "Adjustable Shelf Standards and Supports" Paragraph below specifies standards and knife-type brackets for mounting at rear of shelves, which is most common; second specifies standards and clip-type rests for mounting at ends of shelves.

        - Adjustable Shelf Standards and Supports: [BHMA A156.9, B04102; with shelf brackets, B04112; recess mounted in rear surface] [BHMA A156.9, B04071; with shelf rests, B04081; mounted on sides]. Provide standards extending full height of display case.

        - Dimensions:

          - Height: 4'-0"

          - Width: [4'-0"] [5'-0"] [6'-0"] [8'-0"] [10'-0"] [12'-0"] [16'-0"]

          - Depth: [12"] [16"] [24"].

Select back panel material from one of the two following options.

        - Back Panel: Manufacturer's standard [linoleum] [vinyl-fabric-faced] tackboard panel.

- Delete the following if not required

        - Illumination System: Concealed top-lighting system consisting of fluorescent-strip fixtures. Include lamps and internal wiring with single, concealed electrical connection to building system. Coordinate electrical characteristics with power supply provided.

          - Ballasts: Low-temperature, high-power-factor, low-energy, fluorescent lamp ballasts that comply with Certified Ballast Manufacturers Association standards and carry its label.

            - Electrical Characteristics: Single-phase, [120] [277] volts.

      - BACK TACKBOARD PANELS

        - Linoleum Tackboard Panel: 1/4-inch- (6-mm-) thick, plastic-impregnated-cork sheet factory laminated to 1/4-inch- (6-mm-) thick, fiberboard backing.

        - Vinyl-Fabric-Faced Tackboard Panel: Type II self-healing vinyl over 1/4-inch- (6-mm-) thick, cork sheet factory laminated to 1/4-inch- (6-mm-) thick, medium density fiberboard backing.

- Retain or delete the following, as appropriate

      - MATERIALS

        - Fiberboard: ASTM C208.

Generally, select wither Linoleum or Vinyl Fabric below.

        - Linoleum: 1/4-inch- (6-mm-) thick homogeneous tackable surface material made of primary natural materials consisting of linseed oil, cork, rosin binders and dry pigments mixed and calendared onto a natural jute backing.  The uni-color extends throughout the thickness of the material.

        - Vinyl Fabric: ASTM F793/F793M, Type II, burlap weave; weighing not less than 13 oz./sq. yd. (440 g/sq. m); with flame-spread index of 25 or less when tested in accordance with ASTM E84.

        - Extruded-Aluminum Bars and Shapes: ASTM B221 (ASTM B221M), Alloy 6063.

        - Aluminum Tubing: ASTM B429/B429M, Alloy 6063.

        - Clear Tempered Glass: ASTM C1048, Kind FT, Condition A, Type I, Class 1, Quality Q3, with exposed edges seamed before tempering.

        - Fasteners: Provide screws, bolts, and other fastening devices made from same material as items being fastened, except provide hot-dip galvanized, stainless steel, or aluminum fasteners for exterior applications. Provide types, sizes, and lengths to suit installation conditions. Use security fasteners where exposed to view.

      - FABRICATION

        - Fabricate display cases to requirements indicated for dimensions, design, and thickness and finish of materials.

        - Use metals and shapes of thickness and reinforcing required to produce flat surfaces, and to impart strength for size, design, and application indicated.

        - Fabricate cabinets and door frames with reinforced corners, mitered to a hairline fit, with no exposed fasteners.

        - Fabricate shelf standards plumb and at heights to align shelf brackets for level shelves.

      - GENERAL FINISH REQUIREMENTS

        - Comply with NAAMM/NOMMA 500 for recommendations for applying and designating finishes.

        - Protect mechanical finishes on exposed surfaces from damage by applying a strippable, temporary protective covering before shipping.

        - Appearance of Finished Work: Noticeable variations in same piece are unacceptable. Variations in appearance of adjoining components are acceptable if they are within the range of approved Samples and are assembled or installed to minimize contrast.

      - ALUMINUM FINISHES

Retain finishes in this article to suit Project. If retaining more than one, indicate location of each on Drawings or by inserts.

        - Clear Anodic Finish: AAMA 611, AA-M12C22A31, Class II, 0.010 mm or thicker.

        - Color Anodic Finish: AAMA 611, AA-M12C22A32/A34, Class II, 0.010 mm or thicker.

"Baked-Enamel or Powder-Coat Finish" Paragraph below references AAMA standard for pigmented organic coating on extrusions and panels.

        - Baked-Enamel or Powder-Coat Finish: AAMA 2603, except with a minimum dry film thickness of 1.5 mils (0.04 mm). Comply with coating manufacturer's written instructions for cleaning, conversion coating, and applying and baking finish.

- EXECUTION

      - EXAMINATION

        - Examine walls, with Installer present, for compliance with requirements for installation tolerances, surface conditions of wall, and other conditions affecting performance of the Work.

        - Examine roughing-in for electrical power systems to verify actual locations of connections before installation of illuminated units.

        - Examine walls and partitions for proper backing for display cases.

        - Examine walls and partitions for suitable framing depth if recessed units will be installed.

        - Proceed with installation only after unsatisfactory conditions have been corrected.

      - INSTALLATION

        - General: Install units in locations and at mounting heights indicated on Drawings, or if not indicated, at heights indicated below. Keep perimeter lines straight, level, and plumb. Provide grounds, clips, backing materials, adhesives, brackets, anchors, trim, and accessories necessary for complete installation.

          - Mounting Height: 72 (1830) inches (mm) above finished floor to top of cabinet.

        - Bulletin Boards: Attach units to wall surfaces with concealed clips, hangers, or grounds.

      - ADJUSTING AND CLEANING

        - Adjust doors to operate smoothly without warp or bind and so contact points meet accurately. Lubricate operating hardware as recommended in writing by manufacturer.

        - Touch up factory-applied finishes to restore damaged areas.

END OF SECTION 101200